# Food-intake detection from AIM-2 sensor signals: literature review and model description

Caelan Dunlea, University of Alabama

1 October 2026

# 1	Literature review

Objective measurement of eating behaviour matters for obesity, eating disorders and diabetes research, and self-report is known to be inaccurate and cannot capture in-meal behaviour such as eating rate or pauses (Diou et al. 2022). Wearable sensors that detect chewing, bites or hand gestures have therefore become a central approach, ranging from classical machine learning on hand-crafted features to end-to-end deep learning, with the aims of improving accuracy, reducing false detections and generalising to people outside the training set (Bell et al. 2020). Fontana, Farooq, and Sazonov (2014) introduced the Automatic Ingestion Monitor (AIM), combining jaw-motion, hand-gesture and accelerometer sensors, and reported 89.8% food-intake detection accuracy over 24 hours of free living. Its successor, AIM-2 (Doulah et al. 2021), moved the sensors onto eyeglasses: a temporalis-muscle sensor and a 3-axis accelerometer at 128 Hz. It extracted 38 hand-crafted features per signal, selected nine by minimum-redundancy maximum-relevance, and classified 10 s epochs with a support vector machine, reaching F1 0.818 under leave-one-subject-out validation on 30 participants, as summarised in Table 1. These feature-engineered models work, but their performance is bounded by the features chosen by hand, and they are sensitive to noise and to variation between people.

Table 1. State-of-the-art methods for food-intake and chewing detection, and the two ECG studies whose methods transfer to this problem.

| References | Techniques | Goals | Findings |
| --- | --- | --- | --- |
| Doulah et al. (2021) | AIM-2; hand-crafted features, mRMR selection, SVM | Food-intake detection with passive food-image capture | F1 0.818 (LOSO, 30 participants); camera triggered only during eating |
| Diou et al. (2022) | Review: CNN on in-ear audio windows for chewing, smartwatch IMU for bites | Intake monitoring in free living | CNN F1 0.908 against 0.748 for an SVM; self-supervised learning comparable to supervised |
| Kyritsis, Diou, and Delopoulos (2021) | CNN + LSTM, end to end on raw wrist IMU | Bite and meal detection in the wild | Bite F1 0.923 (LOSO) |
| Ghosh and Sazonov (2022) | Five deep networks (MLP, time-CNN, FCN, ResNet, Inception) on AIM-2 optical and accelerometer time series | Compare deep architectures for food-intake detection | ResNet best, balanced accuracy 93.47% |
| Ghosh et al. (2024) | Wavelet scalogram of accelerometer, fused with food-image recognition | Reduce false positives in free living | Fused F1 80.77%; fusion cuts false positives |
| Stankoski et al. (2021) | Selection of hard negative examples for training | Imbalanced data with imperfect labels | Person-independent F1 0.82 |
| C. Wang et al. (2024) | Temporal CNN + multi-head self-attention | Eating speed from wrist IMU, towards free living | 61 participants, 513 h; eating-speed MAPE 0.110 |
| Vedovelli et al. (2026) | Transformer encoder and other models on smartwatch sensors | Eating-episode recognition | Transformer: sensitivity 0.691, specificity 0.504 (LOSO) |
| Hannun et al. (2019) | 34-layer 1D CNN on raw single-lead ECG | End-to-end arrhythmia classification | F1 0.837, above the average cardiologist (0.780) |
| Ikram et al. (2025) | Transformer on MIT-BIH beats | Five-class arrhythmia classification | Accuracy 97.1%; split not at patient level |

## 1.1	From hand-crafted features to learned representations

To address the limits of hand-crafted features, several groups trained networks directly on the signal. In this literature, a raw input denotes the absence of hand-engineered features rather than of preprocessing: z-normalisation is the only preprocessing in the convolutional time-series baselines of Z. Wang, Yan, and Oates (2017), and the end-to-end bite detector reviewed by Diou et al. (2022) receives preprocessed inertial signals. Papapanagiotou, Diou, and Delopoulos (2017) detected chewing with a 1D convolutional network applied to in-ear audio windows; across window lengths of 1 to 5 s, the 5 s network reached F1 0.908, against 0.748 for a support vector machine on hand-crafted features, although such models “require significantly larger volumes of labeled data” (Diou et al. 2022). Similarly, Kyritsis, Diou, and Delopoulos (2021) trained a CNN feature extractor and an LSTM jointly on raw wrist inertial data, reaching a bite-detection F1 of 0.923 under leave-one-subject-out evaluation. On AIM-2 itself, Ghosh and Sazonov (2022) compared five deep architectures on the optical and accelerometer time series and found the residual network best, with a balanced accuracy of 93.47%. The same pattern holds outside eating: the fully convolutional network of Z. Wang, Yan, and Oates (2017), three blocks of convolution, batch normalisation and ReLU, is a strong baseline across time-series benchmarks.

The closest analogue to this problem is electrocardiography. Hannun et al. (2019) classified arrhythmia with a 34-layer 1D CNN applied end to end to raw single-lead ECG at 200 Hz, and exceeded the average cardiologist (F1 0.837 against 0.780). ECG and the AIM-2 signals share a sampling rate of the same order, a quasi-periodic structure (heart beats at about 1 Hz, chews at 0.94–2.17 Hz (Po et al. 2011)) and the need to recognise short events inside longer recordings. This motivates the first and most important design choice in this work: the model reads the normalised time series, not a spectrum. None of the reviewed studies compares the two representations on the same network, so that comparison is made directly in this work. Ghosh et al. (2024) took the opposite route, converting accelerometer signals to a wavelet scalogram image. A magnitude spectrum says which rhythms are present in a window but discards when they occur, which is the information that separates a bite from background motion; a scalogram keeps a time axis, but at a resolution fixed by the transform rather than learned from the data.

## 1.2	Attention and Transformers

Attention mechanisms let a network weight the parts of its input that carry evidence. Squeeze-and-excitation (Hu, Shen, and Sun 2018) gates feature channels with a small learned function of their global average, at negligible cost. Attention-based pooling (Ilse, Tomczak, and Welling 2018) treats a window as a bag of instances and learns which instances determine its label; for eating, the instances are time steps and a single bite may be the deciding one. Transformers extend this to attention between all time steps. Ikram et al. (2025) applied a Transformer to five-class arrhythmia classification on MIT-BIH, reporting 97.1% accuracy. The reported 80/20 split is of preprocessed samples rather than of patients, so the figure measures performance on new beats from known patients, not on new patients. In eating detection, C. Wang et al. (2024) combined a temporal CNN with multi-head self-attention to measure eating speed from wrist IMU data on 61 participants, while Vedovelli et al. (2026) found that a Transformer encoder on smartwatch data under leave-one-subject-out gave the highest sensitivity (0.691) but a specificity of only 0.504, which is the false-positive problem in its clearest form.

Despite their promise, Transformers carry many more parameters than the attention layers above, and whether they help at a given data size is an empirical question rather than a given. This work therefore uses the two light attention mechanisms by default and treats a Transformer over the convolutional features as a separate experiment, on the same folds.

## 1.3	Supervision, augmentation and negative data

Small annotated datasets call for more supervision per example and for more examples per recording. Multi-task learning (Caruana 1997) improves generalisation by training a shared representation on related tasks; here, counting chews is the related task, since the ground truth already contains chew annotations. This auxiliary task is specific to the present work. Augmentation of wearable sensor data raised accuracy from 77.54% to 86.88% in Parkinson’s disease monitoring (Um et al. 2017); overlapping windows are the simplest form of it. Window length itself trades resolution for context, and has a measurable effect on recognition (Banos et al. 2014). For chewing, which “typically is in the range from 1 to 3 Hz”, a 5 s window was chosen so that “even for the lowest frequency of 1 Hz, a window of 5 s contains at least 5 periods” (Diou et al. 2022); the 8 s windows used here contain at least eight.

False positives are the persistent weakness of eating detectors: many non-eating activities produce jaw and head motion. Walking in particular has a rhythm similar to chewing, 1 to 3 Hz, and causes chewing classifiers “to yield high false-positive detections”, which accelerometer data can be used to remove (Diou et al. 2022). The same work notes that a detector can be tuned “towards higher recall or precision, or a balanced mode, based on the needs of each use-case”. Stankoski et al. (2021) reduced them by deliberately selecting hard negative examples for training, and Ghosh et al. (2024) by fusing the sensor classifier with food-image recognition. Finally, evaluation practice varies widely across the field (Bell et al. 2020). Saeb et al. (2017) showed that record-wise cross-validation, where windows from one person appear in both training and test sets, can massively overestimate accuracy, and that subject-wise validation is the one that matches the use case. The leave-one-subject-out protocol used by Doulah et al. (2021) and Diou et al. (2022) is adopted here for the same reason.

## 1.4	Contribution of this work

- A compact convolutional model with channel attention, temporal attention pooling and an auxiliary chew-count head, reading the normalised AIM-2 optical and accelerometer signals directly from the ETS database: 61 annotated sessions from 43 participants, evaluated at the participant level.

- A direct comparison of time-series and spectral input on the same network, which none of the reviewed studies reports. The time series reaches F1 0.858 against 0.796 and reduces false alarms from 81 to 32 per hour of non-eating.

- A controlled ablation of each design decision, one at a time on identical participant-level folds, separating the components that carry the performance from those that do not.

- Negative results reported with their mechanism: temporal median smoothing and context windows without a target marker both lower F1, because meals at 8 s resolution are punctuated rather than contiguous.

- A measured check of sensor-annotation alignment on every session, replacing an assumed correction with a verified one.

Despite the progress in deep learning for intake monitoring, several challenges persist: limited annotated data, variation between participants in sensor fit and chewing style, false positives from non-eating jaw and head motion, and inconsistent evaluation that makes published results hard to compare. Addressing these requires models that generalise to people they have not seen, evaluated in the way they would be used.

# 2	Model description

The model is a compact one-dimensional convolutional network that reads the normalised AIM-2 sensor signals and outputs the probability that an 8 s window contains eating (Figure 1). It has four parts. A convolutional feature extractor learns its own filters from the time series instead of receiving hand-crafted features or a spectrum. A squeeze-and-excitation gate reweights the learned feature channels in each window. An attention pooling layer weights the time steps that carry the evidence. Finally, the output has two heads: one classifies eating, and one counts chews and is used only during training. The network has 47,282 parameters at inference (Table 2). Table 3 lists every design decision with its reason and the literature it rests on.

Figure 1. Model architecture. Shaded blocks are the two attention mechanisms. The dashed chew-count head is trained jointly and discarded at inference. Output shapes are time steps × features.

Table 2. Layer dimensions. An 8 s window at 128 Hz is 1024 samples; each of the 64 output time steps covers about 0.9 s of signal at 125 ms spacing.

| Layer | Configuration | Output | Parameters |
| --- | --- | --- | --- |
| Input | Acc X, Y, Z, Optical | 1024 × 4 | – |
| Conv block 1 | 32 × k9, stride 2, BN, max-pool 2 | 256 × 32 | 1,312 |
| Conv block 2 | 64 × k9, stride 2, BN, max-pool 2 | 64 × 64 | 18,752 |
| Conv block 3 | 64 × k5, BN | 64 × 64 | 20,800 |
| Channel attention | 64 → 16 → 64, sigmoid | 64 × 64 | 2,128 |
| Attention pooling | 1 × 1 conv, softmax over time | 64 | 65 |
| Dense | 64, ReLU, dropout 0.3 | 64 | 4,160 |
| Eating head | 1, sigmoid | 1 | 65 |
| Total at inference |  |  | 47,282 |
| Chew-count head | 32 → 1, softplus (training only) | 1 | 2,113 |

Table 3. Complete description of the food-intake detection model. Every design decision, the reason for it, and the literature it is grounded in.

| # | Component | Design | Why | Literature |
| --- | --- | --- | --- | --- |
| Data |  |  |  |  |
| 1 | Sensor | AIM-2 eyeglass sensor: 3-axis accelerometer and optical temporalis-muscle sensor, 128 Hz, stored as 8 s packets in the ETS database | Chewing moves the temporalis muscle and the head; both channels carry it, with different artefacts. The original AIM-2 used a flex sensor where this one has the optical sensor | Doulah et al. (2021; Ghosh and Sazonov 2022) |
| 2 | Labels | Eating = bout OR bite annotation at 10 Hz. Window is eating if more than half its annotated samples are eating. Unrecorded time (−1) is dropped, never treated as non-eating | Treating unannotated time as negative would bury the real labels and invent easy negatives | (this work) |
| 3 | Alignment | No time shift. Sensor/annotation lag measured by cross-correlating the optical chewing envelope with the labels: median +0.10 s, all within 0.2 s on the 20 sessions measured first (to be repeated on all 61) | A correction must be measured, not assumed; a residual lag of one packet would corrupt every label edge | (this work) |
| 4 | Window | 8 s (1024 samples), non-overlapping at evaluation | Chewing lies at 1–3 Hz; a 5 s window was chosen for chewing detection so that it holds at least five chews, and AIM-2 used 10 s epochs. An 8 s window holds at least eight chews and matches the packet size. Lengths of 2–16 s are also evaluated | Diou et al. (2022; Doulah et al. 2021; Po et al. 2011) |
| Input |  |  |  |  |
| 5 | Representation | Normalised time series, 1024 samples × 4 channels; no spectrum and no hand-crafted features | In the reviewed studies, features learned from the signal outperformed hand-engineered ones. None compares time-series and spectral input on the same network; that comparison is made here (61 sessions, 5-fold): F1 0.858 against 0.796, and 32 against 81 false alarms per hour of non-eating | Ghosh and Sazonov (2022; Kyritsis, Diou, and Delopoulos 2021; Diou et al. 2022); this work |
| 6 | Normalisation | Per window, per channel: mean removed, z-scored over valid samples only; missing samples set to 0 after scaling | Optical baseline varies by participant and fit on the face; DC level is not information about chewing. A literal −1 against counts near 3000 injects a step at every gap. z-normalisation is likewise the only preprocessing in the standard convolutional time-series baselines | Z. Wang, Yan, and Oates (2017) |
| Network |  |  |  |  |
| 7 | Convolutional trunk | Three 1D conv blocks: 32@k9/s2, 64@k9/s2, 64@k5, each with batch norm and ReLU, max-pool 2 after the first two. Output 64 time steps × 64 features | Learns its own filters from the signal. For chewing detection, a CNN reached F1 0.908 against 0.748 for an SVM on hand-crafted features. Each output step covers about 0.9 s of signal (one chew cycle) at 125 ms spacing | Z. Wang, Yan, and Oates (2017; Ghosh and Sazonov 2022; Diou et al. 2022; Hannun et al. 2019) |
| 8 | Channel attention | Squeeze-and-excitation: global average → dense 16 (ReLU) → dense 64 (sigmoid) → rescale each feature channel | Walking at 1–3 Hz resembles chewing and produces false positives in chewing detectors; the accelerometer is what separates the two. The gate reweights optical against motion channels in every window at negligible cost | Hu, Shen, and Sun (2018; Diou et al. 2022) |
| 9 | Temporal pooling | Attention pooling: one score per time step (1 × 1 conv), softmax over the 64 steps, weighted sum | Global averaging dilutes a 1–2 s bite across 8 s. Attention weights the informative steps, as in multiple-instance learning | Ilse, Tomczak, and Welling (2018) |
| 10 | Classifier | Dense 64 (ReLU), dropout 0.3, sigmoid output | Small head; about 47 k parameters in total at inference | – |
| 11 | Auxiliary head | Chew-count regression (dense 32 → 1, softplus), target scaled by its 99th percentile. Used in training only | Deep models require large volumes of labelled data, and one label bit per window is thin supervision. The chew-count task is specific to this work; the principle is multi-task learning | Caruana (1997; Diou et al. 2022) |
| Training |  |  |  |  |
| 12 | Loss | Binary cross-entropy (weight 1.0) + chew MSE (weight 0.2); class-balanced sample weights computed per fold | Auxiliary loss is a regulariser, not the objective. Eating fraction ranges 0–86% by session, so the rare class changes with the fold | Caruana (1997) |
| 13 | Augmentation | 50% overlapping windows (4 s hop) at training time only | Roughly doubles the training windows at no collection cost; evaluation stays on the non-overlapping grid so no second is scored twice | Um et al. (2017) |
| 14 | Optimisation | Adam, learning rate 0.0003, batch 64, up to 80 epochs, early stopping (patience 12) and learning-rate halving on validation ROC AUC | At 0.001 the validation optimum fell at the first epoch in most folds; the lower rate moves it later. Validation participants are held out from training, never random windows | – |
| Evaluation |  |  |  |  |
| 15 | Validation | Leave-one-subject-out: 43 folds, each holding out one participant with all of their sessions. Inner validation on further held-out participants | Windows of one meal are near-duplicates; any split below participant level leaks and overstates performance. Leave-one-subject-out is the protocol of the AIM-2 and smartwatch intake studies | Doulah et al. (2021; Kyritsis, Diou, and Delopoulos 2021; Diou et al. 2022; Saeb et al. 2017) |
| 16 | Threshold | Chosen on the inner validation participants, never on the test participant: at the centre of the F1 plateau by default, or as the highest recall within a false-alarm budget per hour | The F1 curve is flat near its peak, so its exact argmax on few participants is noise. A detector can be tuned towards recall or precision according to the use case; in free living, false alarms carry the higher cost | Diou et al. (2022) |
| 17 | Post-processing | None. Median smoothing over 3 windows was tested and rejected (F1 −0.079) | Meals are punctuated by pauses; at 8 s the label genuinely switches, and smoothing erases short real bouts | (this work) |
| 18 | Metrics | F1 and balanced accuracy lead; accuracy, precision, recall and false alarms per hour of non-eating reported alongside, pooled over all held-out windows and per participant | Class balance differs by participant; accuracy alone rewards predicting the majority class. False alarms per hour expresses the free-living failure mode directly | Bell et al. (2020) |

## 2.1	Dataset and current performance

The dataset is 61 annotated eating sessions (breakfast, lunch, dinner and snack) from 43 participants, 2 to 55 minutes each: 24.2 hours of annotated recording, of which 9.6 hours is eating. The eating fraction ranges from 0% to 86% by session. Validation is leave-one-subject-out: 43 folds, each holding out one participant with all of their sessions.

Under leave-one-subject-out validation, pooled over all 10,891 held-out non-overlapping windows, the model reaches F1 0.856, accuracy 0.887, balanced accuracy 0.881, precision 0.860, recall 0.853 and specificity 0.909, with 41 false alarms per hour of non-eating. Per-participant F1 has mean 0.836 (standard deviation 0.117, median 0.867), and three participants fall below 0.55. The same configuration under participant-level 5-fold cross-validation gives F1 0.864 and accuracy 0.893, so the estimate is not sensitive to the validation scheme.

The training configurations tested (early-stopping criterion, learning rate, augmentation) differ by no more than 0.006 F1, while replacing the time series with a magnitude spectrum lowers F1 from 0.858 to 0.796 and raises false alarms from 32 to 81 per hour (5-fold). The input representation therefore carries the performance, and the variation between participants is larger than any difference between training configurations.

# References

Banos, Oresti, Juan-Manuel Galvez, Miguel Damas, Hector Pomares, and Ignacio Rojas. 2014. “Window Size Impact in Human Activity Recognition.” Sensors 14 (4): 6474–99. https://doi.org/10.3390/s140406474.

Bell, Brooke M., Ridwan Alam, Nabil Alshurafa, Edison Thomaz, Abu S. Mondol, Kayla de la Haye, John A. Stankovic, John Lach, and Donna Spruijt-Metz. 2020. “Automatic, Wearable-Based, in-Field Eating Detection Approaches for Public Health Research: A Scoping Review.” Npj Digital Medicine 3: 38. https://doi.org/10.1038/s41746-020-0246-2.

Caruana, Rich. 1997. “Multitask Learning.” Machine Learning 28: 41–75. https://doi.org/10.1023/A:1007379606734.

Diou, Christos, Konstantinos Kyritsis, Vasileios Papapanagiotou, and Ioannis Sarafis. 2022. “Intake Monitoring in Free-Living Conditions: Overview and Lessons We Have Learned.” Appetite 176: 106096. https://doi.org/10.1016/j.appet.2022.106096.

Doulah, Abul, Tonmoy Ghosh, Delwar Hossain, Masudul H. Imtiaz, and Edward Sazonov. 2021. “‘Automatic Ingestion Monitor Version 2’ – a Novel Wearable Device for Automatic Food Intake Detection and Passive Capture of Food Images.” IEEE Journal of Biomedical and Health Informatics 25 (2): 568–76. https://doi.org/10.1109/JBHI.2020.2995473.

Fontana, Juan M., Muhammad Farooq, and Edward Sazonov. 2014. “Automatic Ingestion Monitor: A Novel Wearable Device for Monitoring of Ingestive Behavior.” IEEE Transactions on Biomedical Engineering 61 (6): 1772–79. https://doi.org/10.1109/TBME.2014.2306773.

Ghosh, Tonmoy, Yue Han, Viprav Raju, Delwar Hossain, Megan A. McCrory, Janine Higgins, Carol Boushey, Edward J. Delp, and Edward Sazonov. 2024. “Integrated Image and Sensor-Based Food Intake Detection in Free-Living.” Scientific Reports 14: 1665. https://doi.org/10.1038/s41598-024-51687-3.

Ghosh, Tonmoy, and Edward Sazonov. 2022. “A Comparative Study of Deep Learning Algorithms for Detecting Food Intake.” In 44th Annual International Conference of the IEEE Engineering in Medicine and Biology Society (EMBC), 2993–96. https://doi.org/10.1109/EMBC48229.2022.9871278.

Hannun, Awni Y., Pranav Rajpurkar, Masoumeh Haghpanahi, Geoffrey H. Tison, Codie Bourn, Mintu P. Turakhia, and Andrew Y. Ng. 2019. “Cardiologist-Level Arrhythmia Detection and Classification in Ambulatory Electrocardiograms Using a Deep Neural Network.” Nature Medicine 25 (1): 65–69. https://doi.org/10.1038/s41591-018-0268-3.

Hu, Jie, Li Shen, and Gang Sun. 2018. “Squeeze-and-Excitation Networks.” In Proceedings of the IEEE Conference on Computer Vision and Pattern Recognition (CVPR), 7132–41. https://doi.org/10.1109/CVPR.2018.00745.

Ikram, Sunnia, Amna Ikram, Harvinder Singh, Malik Daler Ali Awan, Sajid Naveed, Isabel De la Torre Díez, Henry Fabian Gongora, and Thania Candelaria Chio Montero. 2025. “Transformer-Based ECG Classification for Early Detection of Cardiac Arrhythmias.” Frontiers in Medicine 12: 1600855. https://doi.org/10.3389/fmed.2025.1600855.

Ilse, Maximilian, Jakub M. Tomczak, and Max Welling. 2018. “Attention-Based Deep Multiple Instance Learning.” In Proceedings of the 35th International Conference on Machine Learning (ICML), 80:2127–36. Proceedings of Machine Learning Research.

Kyritsis, Konstantinos, Christos Diou, and Anastasios Delopoulos. 2021. “A Data Driven End-to-End Approach for in-the-Wild Monitoring of Eating Behavior Using Smartwatches.” IEEE Journal of Biomedical and Health Informatics 25 (1): 22–34.

Papapanagiotou, Vasileios, Christos Diou, and Anastasios Delopoulos. 2017. “Chewing Detection from an in-Ear Microphone Using Convolutional Neural Networks.” In 39th Annual International Conference of the IEEE Engineering in Medicine and Biology Society (EMBC), 1258–61. https://doi.org/10.1109/EMBC.2017.8037060.

Po, J. M. C., J. A. Kieser, L. M. Gallo, A. J. Tésenyi, P. Herbison, and M. Farella. 2011. “Time-Frequency Analysis of Chewing Activity in the Natural Environment.” Journal of Dental Research 90 (10). https://doi.org/10.1177/0022034511416669.

Saeb, Sohrab, Luca Lonini, Arun Jayaraman, David C. Mohr, and Konrad P. Kording. 2017. “The Need to Approximate the Use-Case in Clinical Machine Learning.” GigaScience 6 (5). https://doi.org/10.1093/gigascience/gix019.

Stankoski, Simon, Marko Jordan, Hristijan Gjoreski, and Mitja Luštrek. 2021. “Smartwatch-Based Eating Detection: Data Selection for Machine Learning from Imbalanced Data with Imperfect Labels.” Sensors 21 (5): 1902. https://doi.org/10.3390/s21051902.

Um, Terry T., Franz M. J. Pfister, Daniel Pichler, Satoshi Endo, Muriel Lang, Sandra Hirche, Urban Fietzek, and Dana Kulić. 2017. “Data Augmentation of Wearable Sensor Data for Parkinson’s Disease Monitoring Using Convolutional Neural Networks.” In Proceedings of the 19th ACM International Conference on Multimodal Interaction (ICMI), 216–20. https://doi.org/10.1145/3136755.3136817.

Vedovelli, Luca, M. J. Bhuyan, Corrado Lanera, Ileana Baldi, and Dario Gregori. 2026. “Recognition of Eating Episodes via Commercial Smartwatch Sensors.” PLOS Digital Health. https://doi.org/10.1371/journal.pdig.0001539.

Wang, Chunzhuo, T. Sunil Kumar, Walter De Raedt, Guido Camps, Hans Hallez, and Bart Vanrumste. 2024. “Eating Speed Measurement Using Wrist-Worn IMU Sensors Towards Free-Living Environments.” IEEE Journal of Biomedical and Health Informatics 28 (10): 5816–28.

Wang, Zhiguang, Weizhong Yan, and Tim Oates. 2017. “Time Series Classification from Scratch with Deep Neural Networks: A Strong Baseline.” In International Joint Conference on Neural Networks (IJCNN), 1578–85. https://doi.org/10.1109/IJCNN.2017.7966039.